    - The guidance below serves to assist in the identification of objects that may need securing and in the most adequate methods for that

    - The recommended methods are presented in a universally applicable and flexible way

    - It is recommended that crew convert this guidance into a form suiting the particular ship and its available securing equipment

    - A non-exhaustive list of examples of typical heavy objects per onboard departments:

      - objects that should be permanently secured:

- Hotel department - heavy plant pots, sculptures, TVs, cash machines, laundry equipment, slot machines, game machines such as in teen recreation areas

- Shops department - display stands and racks

- Beauty Salon/Spa/Gymnasium department - treatment tables, heavy standalone product displays, treadmills, exercise weight racks, weight lifting machines, display stands and racks

- Entertainment department - pianos, lounge speakers, back-stage scenery equipment, platforms

      - objects that should be secured at all times when not in use:

- Food & Beverage department - store trolleys, baggage cages, plate warmer trolleys, forklift trucks and compressed gas bottles

- Housekeeping/Hotel Department – steward trolleys, lecterns

- Deck department - paint rafts, gangways, deck trash containers, heavy chemicals, paint containers, spare extinguishers

- Security and Medical departments - X-ray machines

- Technical department - cylinder heads, pistons, charge air coolers, gas bottles, heavy chemical containers, heavy A/C coils /equipment, heavy fan impellers, large spare valves, lube oil drums, spare el . motors

      - objects not otherwise secured that should be secured for heavy weather

- Loose objects on display

- Temporary decorations

- Items brought aboard temporarily as part of shows

- Materials/equipment onboard as part of repairs/refurbishment

    - Specific heavy objects that have been identified by the Company that require particular attention

      - Power pack containers:

- Securing to be calculated as compliant with the CSS Code (may be subject to PSC inspection) and the securing devices certificated

- Use of software and relevant printouts such as BV Lash or DNV Lashcon is recommended

- Recommended securing methods and devices for containers:

      - Spare anchor(s)

    - Securing methods and devices:

      - Consideration should be given to:

- the strength and appropriateness of each point of attachment to which the objects are secured (the weakest component in an assembly will be governing):

  - "Maximum securing load" (MSL) is a term used to define the load capacity

  - (typically in kN, where 1kN is approx. 102 kg ) for a device used to secure objects to a ship.

MSL is a proportion of the breaking strength as follows:

  - "Safe Working Load" (SWL) may be substituted for MSL for securing purposes, provided this is equal to or exceeds the strength defined by MSL

- The location of the item onboard as highest accelerations from ship’s motion occur at the ship’s ends: fore, aft and highest decks

- Sliding or Tipping

  - Items which have a low centre of gravity and a large base are likely to slide before they tip over

  - Friction forces affect sliding due to nature of materials in contact:

Hence surfaces should be lined up with materials with high friction coefficients and/or low level securing to prevent sliding

  - Items which have a high centre of gravity and a small base are likely to tip over. Such items in addition to other securing lashings should be fitted with high level lashings and/or propped by shoring at its higher parts

  - The local strength of the deck to ensure no structural damage will occur due to heavier weights

      - Rule of Thumb:

- The total of the MSL values of the securing devices on each side of a unit / object (port as well as starboard) should equal the weight of the object

- Transverse securing angles to the deck should not be greater than 60° and it is important that adequate friction is provided by the use of suitable material. Additional securing at angles of greater than 60° may be desirable to prevent tipping but are not to be counted in the number of securing lashings under the rule-of-thumb

- Some of the securing should have fore-aft component as well as an athwartship component (securing that leads directly fore or aft and have no athwartship component should not be included in the above MSL calculation)

- Example:

  - Weight of an object (ie heavy container on deck) to be secured = 20’200 kg = 20.2 tonnes

  - Total required MSL on EACH side ~ 198.1 kN

VS.

  - WEAKEST / governing of the combined arrangement of Securing devices

    - bulldog grip eye 18mm wire with 13.2 tonnes breaking strength (MLS at 70% =  9.24 tonnes)

    - large D shackle marked with SWL 20 tonne (MSL at 50% = 10.0 tonnes)

    - turnbuckles 500mm, breaking strength 19.0 tonnes (MSL at 50% = 9.5 tonnes)

    - D rings fittings rated at 35 tonnes (MLS at 50% = 17.5 tonnes)

  - -> weakest MSLis of bulldog grip eye wire of 9.24 tonnes or 90.6 kN

  - 2 (two) securing arrangements / devices on each side = 18.48 tonnes or 181.2 kN

?

  - Conclusion =  181.2 kN securing < required MSL 198.1 kN -> insufficient lashing/securing

  - Requirement = minimum 3 (three) devices of MSL 9.24 tonnes to give 27.72 tonnes  or 271.8 kN on each side > 198.1 kN

      - Examples of (combinations of other) securing methods and devices with illustrations:

- A – Latch type gate hook and eye bracket mounted on bulkhead or vertical surface:

- B – Link style chain and eye type brackets mounted on bulkhead or vertical surface:

- C – Bolts through base into tapped holes in welded Deck Pad or Deck anchor device:

- D - Ratchet strap and eye brackets mounted onto bulkhead or vertical surface:

- E – “U” shaped steel bracket round object bolted to bracket on bulkhead or vertical surface or pillar:

- F – Brackets, shackles, wire strops and turnbuckles:

- G – Angle bar welded to structure with studs, bolts and or brackets:

- H – Steel frame mounted to deck and bulkhead with suitable retaining bracket:

- I – Sand filled leather pouches:

- J – Deck mounted bases suitable for object legs secured into the deck with bolts:

- K – Steel bulkhead mounted “U” shaped hooks and ratchet straps:

- L – Angle bar frame with securing points for bolts:

- M – Bracket mounted on bulkhead with wire and spring clip and friction pad under object:

- N – Deck mounted on plinth with securing brackets on front bolted to plinth:

- O- Bracket on object with associated bracket on bulkhead and pin or bolt through brackets:

- P - Rope /Wire Rope– Secured to object and adjacent suitable securing surface

- Q - Permanent securing such as bolting/bracketing to bulkhead:

- R– Steel bracket on object secured to deck with bolts

- S – Spring loaded door mounted latch with deck mounted steel pipe for latch to engage when door in “open” and “closed” position:

- T – Contained in metal rack type shelving system:

- U– Steel frame with studs and butterfly/wing nut:

- V– Plate style base fitted to object and upturned end secured to toe kick or similar surface:

- X – Suction cup and bracket/ratchet strap/chain, etc